Introduction

The scope of the project is to design a car that will be controlled via Bluetooth technology , and will be capable of moving forward, backwards, climbing the ramp inclined to a specific angle, steering, grip the  coffee cup and deliver it to the designated location.

The design objective was to satisfy all the criteria mentioned above. By apply applying the simplified physical- mathematical model the type of motors and wheels size were determined .

It was decided that the car project would have 4 wheels, 2 DC motors, ultrasonic sensor, Bluetooth module, 2 servo motors, gripper, Arduino UNO microcontroller, 12V Battery, motor drive shield, power switch, fuse, LEDs and resistors.

The driving force will come from 2 DC motors and the car will have the rear drive. Ultrasonic sensor will be placed in the front to sense the obstacles, gripper will be placed on the back to pick up the cup . The  battery will be placed between the gripper an the Arduino microcontroller. The rest of the components will be placed on the middle of the car plate. DC motors will be attached to the plate using the L-shape bracket.  The steering system was designed to turn the wheels with the help of a servo motor, which will be placed on the plate. When the servo rotates, the shaft attached to it will move the steering plate, which will turn the wheels right or left depending on the motor direction of rotation.

According to our calculations the approximate weight of the vehicle is around 2.5 kg. Motor speed is 160 RPM( Zhengke Motor,2020). The wheels diameter is 85 mm. These values will allow our car to achieve a speed of 0,71m/s.

The torque produced by the motors is 16,67 N*cm (Zhengke Motor,2020) which will allow us to acquire a force of 70.93 N.

This force is enough for the car to climb a ramp with an angle of 23 degree.

The steering servo motor will produce a Torque of 4 kg/cm (HITEC RCD USA ,2020).  Considering the weight of the car is 2.5 kg and that most of the weight is concentrated on  the Rear Drive. The weight we consider is 0,9kg for the steering mechanism. The mechanical coupling gives us approximately 4cm of lever. F=TL=4kgcm/4 cm=1kg (Erjavec, 2002) Therefore our car will have enough force to steer as F>W.

Freq=mgcosα×μ+mgsinα

Freq. = Ffr. + Fs. (Force of friction + Force of slip)

F req. = = 2.5kg * 9.81m/s^2 * cos(22°) * 0.7 + 2.5kg * 9.81m/s^2 * sin(22°) = 25.1N

Force provided by motors: Fm.

Fm >= Freq. (25.1N)

Fm=2×TR

Then:  R=2×TFm

R = 2 * 39.24 N-cm / 25.1 N = 3.13 cm (maximum allowable wheel radius)

Note:

Our real wheels have a radius of 4.255cm . Performed calculations indicate that the car will no have enough power to climb the ramp. However, it was considered that real motor T is much higher than stated in present data sheet(Zhengke Motor,2020) . The motor model is different , threfore different power reading). Thus, 4.255 cm wheels are chosen and used in further calculations.

Maximum Vehicle Speed

Vmax=RPM×Lwh(2×π×R)

Lwheel  = 2 * pi * 4.255 cm = 26.73 cm/r= 0.2673m/r

Vmax. = 160RPM * 0.2673m/r (per rotation) = 42.77 m/minute ~ 0.71m/s

To hold a cup F holding supposed to be greater than a cup weight.

Wcup (with ping-pong balls) ~ 25g * 9.81m/s^2 / 1000 ~ 0.25N

T=W×d, (distance from the motor shaft to the gripper contact point)

Thold = 0.25N * 7cm = 1.75 N-cm

Actual T of servo motor = 40.21 N-cm

sf=TacThold

sf ~ 22 – Great safety factor ensures Cup is held securely even during acceleration and motion at maximum speed.

Construction Activity

The construction of the car consists of the three important parts.

- The construction of the Steering System

- The construction of the rear drive

- Attachment of the components to the base plate

- Steering System Construction

Our construction activities started with manufacturing parts for the steering mechanism and assembling the parts into one single Steering Assembly.

Our Steering Assembly primarily includes

- 2  L-shape parts referred as Part A

- 2 wheel shafts referred as Part A1

- 2 Holding Plates referred as Part D

- 1 Steering plate referred as Part B

- 1  Motor-steering junction referred as Part C

- 4 Socket Cap screws with washers and nuts

3 weeks are assigned for construction of the Steering mechanism. The activities are performed according to the schedule without delays.

As of date 10.11.2020 (Week4) we started construction of Steering Assembly

Part A manufacturing started and a rough L shape was obrained using Saw and Drill. (Week4, Vladislav Marchenko)

Future activities includes:

- Milling the rough L-shape to acquire the precise dimensions needed (Week5, Vladislav Marchenko)

- Drilling 3 holes in the Part A ( Week 5, Yaroslav Ishutko)

- Manufacturing of Part A1 is required before obtaining the full shape of Part A ( Week 5, Yaroslav Ishutko)

- The press fit of the shaft A1 into Part A (Week 5, Yaroslav Ishutko)

- Manufacturing of parts D,C, B (Week5-6, , Yaroslav Ishutko)

- Assembling of the Steering Mechanism ( Week6, Vladislav Marchenko)

- Drive system construction

Second step of the construction involves attaching the rear drive to the base plate.

The whole construction activities consists of

- Attaching the DC motors to the L brackets ( Week 7, Yaroslav Ishutko)

- Drilling holes on the base  plate to attach the L brackets and the  Steering Assembly(Week 7, Vladislav Marchenko)

- Attaching the L brackets, Steering Assembly to the plate, attaching wheels to the motor’s shafts and to the steering shafts. ( Week 7-8, Vladislav Marchenko)

- Electrical/Power system construction

Third step is to attach the Arduino UNO, Bluetooth module, battery, ultrasonic sensor, gripper, motor drive shield, steering servo moto to the base plate.

Construction consists of:

- Drilling holes on the base plate to attach the components with the fasteners. (Week 8, Yaroslav Ishutko)

- Tighten the components to the plate with bolts and nuts ( Week8, Yaroslav Ishutko)

References

- Zhengke Motor (2020)  ZGB37RG (A6587) Motor Manual

Retrived from : https://e.centennialcollege.ca/d2l/le/content/555717/viewContent/6551457/View

10/17/2020

- HITEC RCD USA  (2020) Hitec HS-422 Servo Motor Manual  Retrieved from : https://www.dfrobot.com/product-152.html

- Erjavec, J. (2002). Manual transmissions and transaxles (3rd ed.). Albany, N.Y: Delmar Thomson Learning.