School Of Engineering Technology and Applied Science (SETAS)

Advanced Manufacturing and Automation Technology (AMAT)

Automation and Robotics Technology

ROBO356- Design and Development Project 1

Section: 009 (Group 2)

Project Name: Remote Control Car

Title of Submission: Progress Report 1

Date: 17.10.2020

Professors:  Alaeddin Bani Milhim

Florin Bratu

Submitted By:

Yaroslav Ishutko (301004764)

Vladislav Marchenko (301005635)

Declaration of Authorship (By Group 2)

We, Yaroslav Ishutko and Vladislav Marchenko, confirm that this breakdown of authorship represents our contribution to the work submitted for assessment and our contribution is our own work and is expressed in our own words. Any uses made within the Technology Report of the works of any other author, separate to the work group, in any form (ideas, equations, figures, texts, tables, programs), are properly acknowledged at the point of use. A list of the references used is included.

Abstract

Remote Control Car project is conducted by Group 2 on ROBO 356 section 009 to successfully design, build, troubleshoot and operate a remotely controlled vehicle capable of riding, steering, climbing low-pitched (up to 23°) ramp, gripping and transporting light objects (small coffee cup). As for week 5, all major aspects of vehicle design are finalized: it is 4-wheel, rear drive machine with a newly-developed “true” steering system controlled by servo motor. Major power components, such as battery, motors and a gripper are shifted to the back, while electronics and signal components (Ultrasonic sensor, Bluetooth module, Arduino UNO microcontroller platform) are located in front. It simplifies wiring, signal processing and allows to avoid extra high-to-low energy buffers (Arduino platform consumes 12V and is capable itself to share 5V for several field devices). Mechanical design description is supported by detailed in-scale 3D construction schematics of the vehicle and its particular components. Vehicle theoretical performance is analyzed, predicted and proved with all necessary formulas, it includes: Driving net force, Maximum vehicle speed, Steering Force, Gripping Force. Finished, ongoing and future construction activities are listed in chronological order and precisely described. As stated, major focus is held on assembling and testing steering mechanism – brand new and raw in design, but promising and unique feature. As a result of ongoing analytical and research process some minor changes were implemented in design besides steering. For example, it was decided to improve gripper design for higher safety and reliability. Finally, related and used references are accurately listed, as well, as descriptions to various formulas.

Table of Contents

- Cover Page											1

- Declaration of Authorship									2

- Abstract 											3

- List of Illustrations										5

- Introduction											6

- Construction Activity

  - Steering System									9

  - Drive system construction								11

  - Electrical/Power system construction						11

- Troubleshooting Activity									12

- Conclusion											15

- Bibliography (List of References)								16

- Glossary of terms 										17

- Formula’s List 										18

List of Illustrations

- Figure_1: Overall mechanical design of the vehicle 					8

- Figure_2: Steering System Design							10

- Figure_3: Initial L-shape design 								13

- Figure_4: Final L-shape design								13

- Figure_5: Gripper Schematic								14

Introduction

The goal of the project is to design a vehicle that will be capable of moving forward, backward, overcoming the inclined  ramp , as well as to steer, grip the coffee cup and deliver it to the designated location. The car must be operated remotely and should be able to complete the task in 4 min. The cup must not touch the floor while being gripped.

The design has some limitations that should be considered while designing the vehicle.

List of  limitations include such parameters:

- Vehicle should be safe to competitors

- Overall vehicle dimensions must not exceed 10 by 16 inches and be able to be carried by one person

- Vehicle wheels must be in a range of 2 to 6 in in diameter

- The only power supply that could be used is 12V Battery

- To propel the vehicle only to specific 12V DC motors were given

- No wood, toy components, glue , tape breadboard,  are allowed

- Vehicle must be self-contained

The design objective was to satisfy all the criteria mentioned above. To achieve the assigned configuration, the simplified physical- mathematical model was used to design the car for the scratch.

First of all it was decided that the vehicle will run on 4 wheels , 2 of which will be directly connected to the shafts of the 12 V DC motors. The other 2 wheel will be used in the steering mechanism. As a control will be used an app on the phone

Secondly, the design was based on the provided materials and parts. For example, the DC motors were meant to be attached to the base via L- bracket of specified shape. The base was supposed to be made of 10 by 16 inch aluminum plate and be able to old the battery an the rest of the components on it. Taking into consideration all the parameters it was decided to design from scratch the unique steering system with shafts, which will be connected to the wheels, and will be operated with help of servo motor to rotate left and right.

Thirdly, to satisfy the goal of the project additional components were obtained. These include: gripper operated by servo motor to grip the cup, Arduino UNO microcontroller as main logic element, Bluetooth module compatible with Arduino to connect to the phone app, Motor drive shield for Arduino to control 2 DC motors, Ultrasonic sensors to measure distance to objects, power switch and fuse for system safety and to avoid shortage in the wiring, LEDs and resistors for indication motion forward, backward or obstacle.

Design Section

Mechanical

The primary idea of mechanical design was to estimate the required base plate dimensions to fit the necessary components : L – brackets, DC motors, steering Mechanism, Battery, Arduino UNO, Bluetooth module, Motor shield, gripper and Ultrasonic sensor. By using the 3D Engineering software Inventor, all parts were drawn accurately to their real- life dimension and placed on the base plate. The estimated length was determined as 12 in. The length for Full steering system including Servo motor, and Steering mechanism was estimated in 5 inch length. By placing the 2 DC motors on the plate opposite to each other it was determined that the required width of the project will no exceed 7 inches.

The mechanical design had to satisfy 3 main features:

- Motion Forward/Backward

- Steering

- Gripping the cup

For motion the Forward/Backward the 2 DC motors were used at the rear, in other words the vehicle was designed to have rear drive. DC motors were connected to the L brackets which itself were connected to the base plate at the rear . All connection were done by Hex cup bolts.

For Steering purpose the Steering Mechanism was designed because it has an advantage of more precise control.

While designing the steering mechanism few criteria were followed such as

- The dimensions of the steering should not exceed 7 by 5 inch;

- Shaft for the steering should be on the same level as shaft of the DC motor to avoid the base plate inclination.

- Steering have to be attached to the base plate mechanically

By considering all of the above it was decided to design 2 Angle blocks of the L- Shape with shafts that will be connected to the base via bolts. From the beneath Angle blocks will be supported by the plate and from top by the base plate itself. To make sure that the Angle blocks steer to the same angle they were both attached to each other by the Steering Angle which will be connected to the servo later. By implying such construction the Angle blocks were lined up and were steering at the same angle when the Steering angle was rotated.

For Gripping the cup, the prototype of the Gripper was ordered and was modified.

The modification of gripper included facing off the unnecessary material from it claws and making the certain bolted connection to the servo motor and later to the plate itself.

After all main components were attached to the base plate, the central of mass was found and the rest of the components were placed to achieve equal weight load of rear drive and the steering mechanism. For such arrangement the heaviest component 12 V battery was placed between these two points.

All of the components were attached to the base plate by bolts except for the LEDs.

They were fitted in a specific bracket and then the bracket was tightly fitted to the base plate holes for the LEDs.

Two LEDs are present at the front and at the rear side for indication of Forward and Backward motion respectively.

The ultrasonic sensor was mounted  at the front on holding bracket first and then attached to a 7 inch long holding panel that was attached to the base plate.

The Gripper which consists of Gripper and servo motor was decided to mount at the rear.

The battery was placed between Gripper and Arduino UNO, motor shield was placed at the top of the Arduino microcontroller as well as the Bluetooth module. Servo motor for the steering was placed between Arduino and ultrasonic sensor.

The secured mounting of battery and steering servo were achieved using holding plates tightened to the base plate by bolts foe each of them.

Switch and Fuse were placed closely to battery to avoid long wiring.

For more information regarding Mechanical Design please refer to the A

- Motor speed is 160 RPM (Zhengke Motor, 2020). The diameter of chosen wheels is 45 mm. These values will allow our car to achieve a maximum speed of 0,71m/s.

Vmax=RPMmax×L(2×π×R)

Lwheel = = 0.2673m

Vmax. = 160RPM*0.2673m (per rotation) = 42.77 m/minute ~ 0.71

- According to our calculations the approximate weight of the vehicle is around 2.5 kg. The torque produced by the motors is 39.24N*cm (Zhengke Motor, 2020)   which will allow us to acquire a force of 34.8 N.

Freq. – force required to climb a ramp

Freq. = Ffr. + Fs. (Force of friction + Force of slip)

Freq=mgcosa×n+mgsina

F req.= 2.5kg * 9.81m/s^2 * cos(23°) * 0.7 + 2.5kg * 9.81m/s^2 * sin(23°) = 25.39N

Fm - force provided by motors

Fm=2×T/R

Fm. = 2 * 39.24 N-cm /2.255cm = 34.80 N

This force is enough for the car to climb a ramp with an angle of 23 degree.

(Note* Actual Torque provided by DC motors, and, as a result, Motor Force is significantly higher than stated in manual)

- The steering servo motor will produce a Torque of 4 kg/cm (HITEC RCD USA, 2020).  Considering the weight of the car is 2.5 kg and that most of the weight is concentrated on the Rear Drive. The weight we consider on steering wheels is 0,7kg. The mechanical coupling gives us approximately 4cm of lever.

F=TL=4kgcm/4 cm=1kg (Erjavec, 2002)

Therefore our car will have enough force to steer as F>W.

- To hold and transport a coffee cup securely F holding supposed to be greater than a cup weight.

Wcup (with ping-pong balls) ~ 25g * 9.81m/s^2 / 1000 ~ 0.25N

T=W×d, (distance from the motor shaft to the gripper contact point)

Thold = 0.25N * 7cm = 1.75 N-cm

Actual T of servo motor = 40.21 N-cm

sf=TacThold

sf ~ 22 – Great safety factor ensures Cup is held securely even during acceleration and motion at maximum speed.

(Note* Presented calculations are also mentioned and justified in Milestone 2)

Construction Activity

The construction of the car consists of the three important parts.

- The construction of the Steering System

- The construction of the rear drive

- Attachment of the components to the base plate

- Steering System Construction

Our construction activities started with manufacturing parts for the steering mechanism and assembling the parts into one single Steering Assembly.

Our Steering Assembly primarily includes

- 2  L-shape parts referred as Part A

- 2 wheel shafts referred as Part A1

- 2 Holding Plates referred as Part D

- 1 Steering plate referred as Part B

- 1  Motor-steering junction referred as Part C

- 4 Socket Cap screws with washers and nuts

3 weeks are assigned for construction of the Steering mechanism. The activities are performed according to the schedule without delays.

As of date 10.11.2020 (Week4) we started construction of Steering Assembly

Part A manufacturing started and a rough L shape was obrained using Saw and Drill. (Week4, Vladislav Marchenko)

Future activities includes:

- Milling the rough L-shape to acquire the precise dimensions needed (Week5, Vladislav Marchenko)

- Drilling 3 holes in the Part A ( Week 5, Yaroslav Ishutko)

- Manufacturing of Part A1 is required before obtaining the full shape of Part A ( Week 5, Yaroslav Ishutko)

- The press fit of the shaft A1 into Part A (Week 5, Yaroslav Ishutko)

- Manufacturing of parts D,C, B (Week5-6, , Yaroslav Ishutko)

- Assembling of the Steering Mechanism ( Week6, Vladislav Marchenko)

- Drive system construction

Second step of the construction involves attaching the rear drive to the base plate.

The whole construction activities consists of

- Attaching the DC motors to the L brackets ( Week 7, Yaroslav Ishutko)

- Drilling holes on the base  plate to attach the L brackets and the  Steering Assembly(Week 7, Vladislav Marchenko)

- Attaching the L brackets, Steering Assembly to the plate, attaching wheels to the motor’s shafts and to the steering shafts. ( Week 7-8, Vladislav Marchenko)

- Electrical/Power system construction

Third step is to attach the Arduino UNO, Bluetooth module, battery, ultrasonic sensor, gripper, motor drive shield, steering servo moto to the base plate.

Construction consists of:

- Drilling holes on the base plate to attach the components with the fasteners. (Week 8, Yaroslav Ishutko)

- Tighten the components to the plate with bolts and nuts ( Week8, Yaroslav Ishutko)

Troubleshooting Activities

Due to ongoing analysis of initial mechanical design and following troubleshooting in terms of optimization of car performance a number of changes were made. It included 1 major mechanical design change and 2 supplementary changes not reported in Milestone 2.

- Development of “true” steering system

It was decided from the beginning to have rare drive car, meaning that both power wheels attached to 2 DC motor would be located on the back of the car, rather than in front. It satisfied our strategy to separate low-power (signal) and high-power (energy) systems. While Battery would be located closer to motors, powering them with 12V DC, major electrical components (Ultrasonic sensor, Bluetooth module ) would be located in front and powered with 5V by Arduino motherboard located in center-front part of the car. Thus, we solved 2 problems: firstly, we simplified wiring, secondly, we avoided using extra voltage step-down transition.

However, after comprehensive analysis it was decided to give up on steering by speed difference of power wheels. Rare drive and shifted back center of gravity made speed difference steering unreliable method.  A completely new steering method had to be developed.

After gathering basic information about steering systems and car transmissions new unique steering system for Remote Control Car was developed. It came down to an idea of controlling wheels with servomotor. 2 L-shaped parts (Part A) are connected to wheels by pressing wheel shaft into one of its ends. L-shaped parts can rotate on shafts of a frame (Part D) connected to car bottom. Special steering plate moved by a servomotor control rotation of L-shaped parts and, accordingly, wheels.

(To find out more about its operation and structure refer “XXX”and Figure X)

New system provides increased controllability of rear drive and satisfies initial design limitations.

- Minor change in a design of a steering system component

Initial design of L-shaped part of a newly-developed steering system included variety of different dimensions and sizes for different part components. It was done to ensure minimal weight and ideal balance in center shaft. After discussing initial design with faculty it was decided to develop more uniform and simplified part design contributing to reliability and simplicity of construction. Furthermore, initial idea of machining the wheel shaft within the solid part was rejected. Instead it was decided to drill a hole on the end of the part and press fit a separate wheel shaft into it (assembly). Aluminum was chosen as a construction material due to its low density (as for metals) and satisfactory rigidity.

Initial L-shape design vs revised design

Initial L-shape design, Figure_3

- Minor Gripper Modification

Since new steering system required a lot of time and resources to be designed and constructed it was decided, in terms of time and resources management, to find most universal and reliable gripper solution. A standard Actobotic gripper, compatible with all standard servo motors, was chosen and order from one of suppliers. It also was assured that gripper is large enough to grab a small coffee cup – maximum width of gripper is 4.20” with total diameter of a cup no larger than 3-3.5”.

However, after analysis of  Actobotic Gripper shape, it was determined  to cut down and polish internal side of gripper so its shape would be ideally suitable for transportation of a specific coffee cup and its internal teeth (on the middle) are removed so cup would not be damaged in any case (safety). Rubber material of the gripper was defined as a tolerant for such deformations

Gripper Schematic, Figure_5

Internal Teeth

Note: original image was retrieved from: www.robotshop.com/ca/en/actobotics-perpendicular-standard-gripper-kit-b.html

c

Conclusion

- Vehicle performance expectations are clearly stated: moving forward, backwards, climbing inclined ramp, steering, gripping the coffee cup and delivering it to the designated location while controlled remotely via Bluetooth.

- Final mechanical design of the vehicle is fully described: 4 wheels, rare drive, “true” steering system controlled with servo motor, physical division of power and signal components.

- Multiple supporting 2D, 3D vehicle/components schematics are provided

- Vehicle performance expectations towards its Driving net force, Maximum vehicle speed, Steering Force, Gripping Force are described and proved by realistic calculations based on engineering formulas and data sheet information. Formulas also have an extra detailed description

- Construction activities for the past, present and future time periods are listed in chronological order and clearly described with division of responsibilities between group members.

- Major mechanical and supplementary Troubleshooting activities that have been conducted are listed, justified and described in-detail.

- All used References are mentioned and listed in alphabetical order

Bibliography

- Erjavec, J. (2002). Manual transmissions and transaxles (3rd ed.). Albany, N.Y: Delmar Thomson Learning.

- HITEC RCD USA.  (2020). Hitec HS-422 Servo Motor Manual

Retrieved from: https://www.dfrobot.com/product-152.html

- Roboshop. Actobotics Perpendicular Standard Gripper Kit B

Retrieved from: https://www.robotshop.com/en/actobotics-perpendicular-standard-gripper-kit-b.html

- Zhengke Motor. (2020).  ZGB37RG (A6587) Motor Manual

Retrieved from: https://www.zhengkemotor.com/product/zhengkemotor_product_Dc_Gear_Motor_37mm_ZGA37RG_ZGB37RG.html

Glossary of Terms

Bluetooth – technology of wireless data exchange between fixed and mobile devices.

Microcontroller – compact integrated electronic circuit capable of conducting complex logical and mathematical operations upon receiving input data. In other words – compact and simplified micro computer.

Motor Shield/Driver – a device or a module that is designed to easily and accurately control multiple motors and their parameters (speed, direction, torque and position). Can be controlled by microcontroller, PLC, PC.

Rear drive or Rear-wheel-drive – form of vehicle transmission at which only rare wheels are powered and driven by motor/s.

Shaft – round rotating machine element that transmits output power.

Formula’s list

Force exerted by servo motor for steering:

F=TL

Force required for the car to climb the ramp:

Freq=mgcosα×μ+mgsinα

Force 2 DC motors can provide:

Fm=2×TR

Maximum Speed of the vehicle:

Vmax=RPM×Lwh(2×π×R)

Torque required to be produced by servo motor to hold the cup:

T=W×d

Safety factor for the gripper:

sf=TacThold